FCN-2 Mock Answers

- Which one of the following is a Class C IP address? *

10.10.14.118

135.23.112.57

191.200.199.199

204.67.118.54

Correct answer

204.67.118.54

- You are asked to evolve a TCP/IP addressing scheme for your Organization. How many network numbers (subnet number) must you allow when you design the network for your organization? *

One subnet for each Host

One for each subnet, one for each WAN connection

One for each network card

One for each WAN connection only

Correct answer

One for each subnet, one for each WAN connection

- You have an IP of 156.233.42.56 with a subnet mask of 7 bits. How many hosts and subnets are possible assuming that subnet 0 is not used? *

126 subnets and 510 hosts

128 subnets and 512 hosts

510 subnets and 126 hosts

512 subnets and 128 hosts

Correct answer

126 subnets and 510 hosts

- You find that one of the serial interfaces on your router is not forwarding traffic. You have given "show ip interfaces" command. In the response, you observed the following entries against that interface:Serial 0 is up, line protocol is down. What is the likely problem? *

There is a problem at physical layer

There is a problem exchanging the routing information

The link was administratively down

There is a problem with the route both at physical and protocol layers

Correct answer

There is a problem exchanging the routing information

- A client workstation is connected to a DHCP server through a switch and a router, The client machine has been able to access the network resources without any problem. However, after the recent configuration change on the DHCP server, the client workstation is not able to access any network resources. What is the likely solution. *

Clear arp cache on the router

Use ipconfig command with /release and /renew options

Reset switch MAC address table

Issue a tracert command to DHCP server

Correct answer

Use ipconfig command with /release and /renew options

- Two sub layers of OSI Data Link layer are which of the following? *

Logical Link Control, Data Link Control

Media Access Control, Physical Layer Control

Logical Link Control, Media Access Control

Data Link Control, Physical Layer Control

Correct answer

Logical Link Control, Media Access Control

- Match the following 1. Segments A. Associated with Data Link Layer 2. Packets B. Associated with Network Layer 3. Frames C. Associated with Transport Layer *

1--->A; 2---->C; 3 --->B

1--->C; 2---->A; 3 --->B

1--->C; 2---->B; 3 --->A

1--->A; 2---->B; 3 --->C

Correct answer

1--->C; 2---->B; 3 --->A

- Match the following A. Repeaters 1. Data Link Layer B. Bridges 2. Network Layer C. Routers 3. Physical Layer *

A --->2, B--->3, C--->1

A--->3, B---->1, C---->2

A--->3,B----->2, C---->1

A---->1, B---->2, C---->3

Correct answer

A--->3, B---->1, C---->2

- Which of the following is true of a Switch? *

Switches forward packets based on IP address.

Switches forward packets based on MAC address.

Switches forward packets based on IPX address.

Switches forward packets based on Layer3 protocol.

Correct answer

Switches forward packets based on MAC address.

- Why is it difficult for routing loops to occur in networks that use link-state routing? *

A.Each router builds a simple view of the network based on hop count.

B.Routers flood the network with LSAs to discover routing loops.

C.Each router builds a complete and synchronized view of the network.

D.Routers use hold-down timers to prevent routing loops.

Correct answer

C.Each router builds a complete and synchronized view of the network.

- Which database or table must be identical on all link-state routers within an area in order to construct an accurate SPF tree? *

A.routing table

B.adjacency table

C.link-state database

D.neighbor table

E.topology database

Correct answer

C.link-state database

- What feature do modern link-state protocols provide to minimize processing and memory requirements? *

A.splitting routing topologies into smaller areas

B.assigning lower process priorities to route calculations

C.using update timers to restrict routing updates

D.strict split horizon rules to reduce routing table entries

Correct answer

A.splitting routing topologies into smaller areas

- What is the purpose of the DHCP server? *

A – to provide storage for email

B – to translate URLs to IP addresses

C – to translate IPv4 addresses to MAC addresses

D – to provide an IP configuration information to hosts

Correct answer

D – to provide an IP configuration information to hosts

- How is the message sent from a PC2 when is first powers on and attempts to contact the DHCP Server? *

A – Layer 3 unicast

B – Layer 3 broadcast

C – Layer 3 multicast

D – Without any Layer 3 encapsulation

Correct answer

B – Layer 3 broadcast

- What is the default behavior of R1 when PC1 requests service from DHCP server? *

A – Drop the request

B – Broadcast the request to R2 and R3

C – Forward the request to R2

D – Broadcast the request to R2, R3 and ISP

Correct answer

A – Drop the request

- How does a DHCP server dynamically assign IP address to host? *

A. Addresses are allocated after a negotiation between the server and the host to determine the length of the agreement.

B. Addresses are assigned for a fixed period of time. At the end of period, a new quest for an address must be made, and another address is then assigned.

C. Addresses are leased to host. A host will usually keep the same address by periodically contacting the DHCP sever to renew the lease.

D. Addresses are permanently assigned so that the host uses the same address at all times.

Correct answer

C. Addresses are leased to host. A host will usually keep the same address by periodically contacting the DHCP sever to renew the lease.

- which IP range is 69.254.0.1 to 169.254.255.254 / 255.255.0.0 *

A. Aging

B. TCP

C. AIPAA

Correct answer

C. AIPAA

- _____ is used to determine the IP address of a computer from its FQDN *

A. Forward Lookup Zone

B. Forwarder

C. Reverse Lookup Zone

D. Normal DHCP Scope

Correct answer

A. Forward Lookup Zone

- What is an example of a MAC address? *

A. Az32:6362:2434

B. BA:281x:8288

C. 101:354:665:8734:ffd6:8023

D. A625:cbdf:6525

Correct answer

D. A625:cbdf:6525

- Which command does not show that two devices are failing to route packets between them successfully? *

A. show interface

B. trace

C. telnet

D. ping

Correct answer

A. show interface

- What range of addresses can be used in the first octet of a Class B network address? *

A. 1–126

B. 1–127

C. 128–190

D. 128–191

E. 129–192

F. 192–220

Correct answer

D. 128–191

- How many bytes is an Ethernet address? *

A. 3

B. 4

C. 5

D. 6

E. 7

F. 8

Correct answer

D. 6

- What protocol is used to find the hardware address of a local device? *

A. RARP

B. ARP

C. IP

D. ICMP

E. BootP

Correct answer

B. ARP

- Which of the following describes a full-duplex transmission? *

A. Data transmission in only one direction

B. Data transmission in both directions, but only one way at a time

C. Uses a single cable

D. Uses a point-to-point connection from the transmitter of the transmitting station to the receiver of the receiving station

Correct answer

D. Uses a point-to-point connection from the transmitter of the transmitting station to the receiver of the receiving station

- Which of the following is not part of the data link layer? *

A. Determines network topology

B. Performs flow control

C. Performs physical addressing

D. Terminates a session

E. Transports data across the physical link

Correct answer

D. Terminates a session

- Which of the following protocols is an example of an exterior gateway protocol? *

A. BGP

B. RIP

C. EIGRP

D. OSPF

Correct answer

A. BGP

- Which command will wipe out all current NAT translations in the NAT table? *

Router#no ip nat translation

Router#clear ip nat translation

Router#clear ip nat translation *

Router#clear ip nat

Correct answer

Router#clear ip nat translation *

- When configuring dynamic NAT, why must you configure an access list? *

The access list allows incoming access from outside global addresses.

The access list allows outgoing access from inside local addresses.

The access list allows outgoing access from outside local addresses.

The access list allows outgoing access from inside global addresses.

Correct answer

The access list allows outgoing access from inside local addresses.

- Which command will allow you to view the current number of active NAT translations on the router? *

Router#show ip nat translations

Router#show ip nat summary

Router#show ip nat status

Router#show ip nat statistics

Correct answer

Router#show ip nat statistics

- Which command will allow you to view the NAT translations active on the router? *

Router#show ip nat translations

Router#show nat translations

Router#debug ip nat translations

Router#show translations nat

Correct answer

Router#show ip nat translations