FCN MOCK 3 Questions with Answers

- Which is a benefit of using TACACS+ for authentication of users? *

It is an open standard.

It encrypts the password of users.

It supports authenticating a user to a subset of commands.

It supports authenticating a user to a length of time.

Correct answer

It supports authenticating a user to a subset of commands.

- Which is an authentication protocol for AAA servers to secure Telnet authentication? *

802.1x

TACACS+

AD

EAP

Correct answer

TACACS+

- Which server will centralize authentication for all Cisco routers and switches? *

Active Directory server

AAA server

802.1x server

Terminal server

Correct answer

AAA server

- What is the default encryption method for passwords, when you configure a line password? *

MD5

SHA-128

SHA-256

Clear text

Correct answer

Clear text

- On which network plane would a routing protocol perform? *

Data plane

Control plane

Management plane

Routing plane

Correct answer

Control plane

- What is an application program interface (API)? *

An API is a program that allows for data transfer.

An API is a programming language for network programmability.

An API is a programming interface or standard allowing one program to communicate with another program.

An API allows for programs to be virtualized.

Correct answer

An API is a programming interface or standard allowing one program to communicate with another program

- Which statement is correct about the software-defined network (SDN) controller? *

The SDN controller configures the management plane of network devices.

The SDN controller monitors data plane traffic.

The SDN controller replaces the control plane of the SDN.

The SDN controller complements the control plane of the SDN.

Correct answer

The SDN controller replaces the control plane of the SDN.

- Which network plane is used for Spanning Tree Protocol (STP)? *

Data plane

Control plane

Management plane

Switch plane

Correct answer

Control plane

- Which version of Simple Network Management Protocol (SNMP) supports the Inform SNMP message? *

SNMP version 1

SNMP version v2

SNMP version 2c

SNMP version 3

Correct answer

SNMP version 2c

- Which command sequence will allow only traffic from network 185.64.0.0 to enter interface s0? *

A. access-list 25 permit 185.64.0.0 255.255.0.0 ; int s0 ; ip access-list 25 out

B. access-list 25 permit 185.64.0.0 255.255.0.0 ; int s0 ; ip access-group 25 out

C. access-list 25 permit 185.64.0.0 0.0.255.255 ; int s0 ; ip access-list 25 in

D. access-list 25 permit 185.64.0.0 0.0.255.255 ; int s0 ; ip access-group 25 in

Correct answer

D. access-list 25 permit 185.64.0.0 0.0.255.255 ; int s0 ; ip access-group 25 in

- Which is true about VLAN Trunk Protocol? Select any 2 *

A. VTP is statically configured and no built in intelligence is available.

B. VTP provides intelligence for configuring switches across the network.

C. VTP is not designed to distribute information across the switch fabric.

D. VTP is designed to distribute information across the switch fabric.

Correct answer

B. VTP provides intelligence for configuring switches across the network.

D. VTP is designed to distribute information across the switch fabric.

- What is true about static VLANs? *

A. The VLAN is configured by a TFTP server.

B. It automatically notify a new network user.

C. The administrator assigns VLAN by port.

D. Static VLAN are more appropriate when the switch fabric becomes more complex.

Correct answer

C. The administrator assigns VLAN by port

- Which database or table must be identical on all link-state routers within an area in order to construct an accurate SPF tree? *

A. routing table

B. adjacency table

C. link-state database

D. neighbor table

E. topology database

Correct answer

C. link-state database

- Which of the following is the correct flow of routines for a router startup? *

A. load bootstrap, load IOS, apply configuration

B. load bootstrap, apply configuration, load IOS

C. load IOS, load bootstrap, apply configuration, check hardware

D. check hardware, apply configuration, load bootstrap, load IOS

Correct answer

A. load bootstrap, load IOS, apply configuration

- When a switch receives a frame and the source MAC address is not found in the switching table, what action will be taken by the switch to process the incoming frame? *

A. The switch will request that the sending node resend the frame.

B. The switch will issue an ARP request to confirm that the source exists.

C. The switch will map the source MAC address to the port on which it was received.

D. The switch ends an acknowledgement frame to the source MAC of this incoming frame.

Correct answer

C. The switch will map the source MAC address to the port on which it was received.

- If a network administrator enters these commands on a switch, what will be the result?

Switch1(config-line)#line console 0

Switch1(config-line)#password cisco

Switch1(config-line)#login

A.to secure the console port with the password "cisco"to deny

B. access to the console port by specifying 0 lines are available

C. to gain access to line configuration mode by supplying the required

password

D. to configure the privilege exec password that will be used for remote

Access

Correct answer

D. to configure the privilege exec password that will be used for remote

Access

- What are two ways to make a switch less vulnerable to attacks like MAC address flooding, CDP attacks, and Telnet attacks? *

A. Enable CDP on the switch.

B. Change passwords regularly.

C. Turn off unnecessary services.

D. Enable the HTTP server on the switch.

E. Use the enable password rather than the enable secret password.

Correct answer

B. Change passwords regularly.

C. Turn off unnecessary services.

- The Internet Control Message Protocol occurs at what layer of the seven layer model? *

A.) Physical

B.) Transport

C.) Session

D.) Datalink

E.) Presentation

F.) Network

Correct answer

F.) Network

- Which type of switching reads in the entire frame before forwarding it? *

A.) Pause-and -forward

B.) Store-and-Forward

C.) Inverse ARP

D.) Fast Forward

E.) Cut-Through

Correct answer

B.) Store-and-Forward

- The ……… protocol allows the administrator to assign a cost, called the metric, to each route. *

A. OSPF

B. RIP

C. BGP

D. BBGP

Correct answer

A. OSPF

- The Open Shortest Path First(OSPF) protocol is an intra domain routing protocol based on …….. routing *

A. distance vector

B. link state

C. path vector

D. non distance vector

Correct answer

B. link state

- An/A ……….routing scheme is designed to enable switches to react to changing traffic patterns on the network. *

A. static routing

B. fixed alternative routing

C. standard routing

D. dynamic routing

Correct answer

C. standard routing

- An area is …. *

A. part of an AS

B. composed of at least two AS

C. another term for an AS

D. composed more than two AS

Correct answer

A. part of an AS

- When a direct delivery is made, both the deliverer and receiver have the same …. *

A. routing table

B. host id

C. IP address

D. Net id

Correct answer

D. Net id

- In OSPF, a ……… link is a network with several routers attached to it. *

A. point-to-point

B. transient

C. stub

D. multipoint

Correct answer

B. transient

- An example of the routing algorithm is … *

A. TELNET

B. TNET

C. ARPANET

D. ARNET

Correct answer

C. ARPANET

- The Enhanced Interior Gateway Routing Protocol(EIGRP) is categorized as a …….. *

A. Distance vector routing protocols

B. Link state routing protocols

C. Hybrid routing protocols

D. Automatic state routing protocols

Correct answer

C. Hybrid routing protocols

- ………. was originally developed to provide a loop-free method of exchanging routing information between autonomous systems. *

A. OSPF

B. EIGRP

C. BGP

D. RIP

Correct answer

C. BGP

- Logical partitioning of the network, authentication and faster convergence rate are the advantages of …. *

A. OSPF

B. EIGRP

C. BGP

D. RIP

Correct answer

A. OSPF

- Which of the following is/are the benefits provided by EIGRP? i) Faster convergence ii) partial routing updates iii) High bandwidth utilization iv) Route summarization *

A. i, iii and iv only

B. i, ii and iii only

C. ii, iii and iv only

D. i, ii and iv only

Correct answer

D. i, ii and iv only