#### VPN Introduction

| 1. | What type of VPN connects two different companies together? - Intranet - Internet - Extranet - Remote access |  |
| --- | --- | --- |
| 2. | HMAC functions perform what function in a VPN solution? - Confidentiality - Packet integrity and authentication - Identity validation - Antireplay |  |

Answers

| 1. | -  C. An extranet connects your company to another company you do business with. -  A connects two locations together from the same company, like a branch office to a corporate office. B is a public network. D connects users or a (SOHO) to a corporate office. |
| --- | --- |
| 2. | -  B. HMAC functions verify that packets haven't been tampered with and are coming from a trusted source. -  A is accomplished through encryption. C verifies a peer's identity, like using a preshared key or digital certificate. D checks for replaying of valid packets. |

#### Cisco VPN Products

| 3. | What terminates VPNs at a primary location? - Router - ASA - Head-end device - Software client |  |
| --- | --- | --- |
| 4. | What type of device does Cisco recommend for site-to-site connections? - 3000 concentrator - PIX - ASA - Cisco IOS router |  |

Answers

| 3. | -  C. A head-end VPN device terminates VPNs at a primary location. -  A and B could be head-end devices, or could be SOHO or branch office devices. D is used for remote access. |
| --- | --- |
| 4. | -  D. Cisco recommends a router for site-to-site connections. -  A and C are recommended for remote access. B is no longer sold by Cisco and lacks many of the L2L features that Cisco routers support. |

#### VPN Design Guide

| 5. | What wizard in SDM can be used to assist in choosing a VPN? - VPN Design Guide - IPSec Wizard - VPN Recommendation Guide - IPSec Guide |  |
| --- | --- | --- |

Answers

| 5. | -  A. The VPN Design Guide Wizard in SDM can be used to assist in choosing a VPN. -  B is used to build a VPN. C and D are nonexistent wizards in VPN. |
| --- | --- |