#### Site-to-Site Preparation

| 1. | Enter the protocol and port that must be specified within an interface ACL to allow a management connection to be terminated on the router: _________________. |  |
| --- | --- | --- |

Answers

| 1. | -  The protocol and port that must be specified within an interface ACL to allow a management connection to be terminated on the router is UDP 500. |
| --- | --- |

#### ISAKMP/IKE Phase 1 Configuration

| 2. | If you omit a parameter in an ISAKMP/IKE Phase 1 policy, a default value is used. Which of the following is not a default value? - MD5 - DES - DH group 1 - RSA signatures |  |
| --- | --- | --- |
| 3. | You need to create an ISAKMP/IKE Phase 1 policy that uses the following parameters: preshared keys, using a preshared key of abc123 for peer 192.1.1.1, 3DES, MD5, DH key group 2, and a lifetime of one hour. Use a policy number of 100. Enter the commands to accomplish this: ___________. |  |

Answers

| 2. | -  A. SHA-1 is the default, not MD5, if the HMAC function is omitted from a Phase 1 policy. -  Answers B, C, and D are default values, if omitted. |
| --- | --- |
| 3. | -  Here is the configuration to set up the policy: - Router(config)# crypto isakmp policy 100 - Router(config-isakmp)# authentication pre-share - Router(config-isakmp)# encryption 3des - Router(config-isakmp)# group 2 - Router(config-isakmp)# hash md5 sha - Router(config-isakmp)# lifetime 3600 - Router(config-isakmp)# exit - Router(config)# crypto isakmp key 0 abc123 address 192.1.1.1 |

#### ISAKMP/IKE Phase 2 Configuration

| 4. | What is the default lifetime of data SAs, if not overridden? - 300 seconds - 600 seconds - 3,600 seconds - 86,400 seconds |  |
| --- | --- | --- |
| 5. | What is the default mode for a transform set, if omitted? - Transport - Transform - Symmetrical - Tunnel |  |
| 6. | Which of the following is not required in a crypto map entry? - Crypto ACL - Router's local IP address - Transform set name - Connection method: manual or dynamic (ISAKMP/IKE) |  |
| 7. | Which of the following traffic is dropped when using static crypto maps and IPSec? - Inbound traffic that is protected and matches a crypto ACL entry - Inbound traffic that doesn't match a crypto ACL entry - Inbound traffic that matches a crypto ACL entry and is in clear text - Outbound traffic that matches a crypto ACL and is already protected by IPSec |  |

Answers

| 4. | -  C. The default lifetime of data SAs, if not overridden, is 3,600 seconds. -  A and B are too small. D is the default value for the management connection. |
| --- | --- |
| 5. | -  D. The default mode for a transform set, if omitted, is tunnel mode. -  A is used for host-to-host connections and is not the default. B and C are nonexistent transform set modes. |
| 6. | -  B. The remote peer's IP address, not the local router's address, must be specified in a crypto map entry. -  Answers A, C, and D are required. |
| 7. | -  C. Inbound traffic that matches a crypto ACL entry and is in clear text is dropped by the router. -  A is verified and decrypted by the router. B is not processed by IPSec. D is protected by IPSec again. |

#### Examining and Troubleshooting Connections

| 8. | What state will a management connection be in if it is successfully set up? - MM_NO_STATE - QM_IDLE - MM_SA_SETUP - MM_KEY_AUTH |  |
| --- | --- | --- |
| 9. | Which output from the debug crypto isakmp command indicates a failed policy negotiation? - %CRYPTO-6-IKMP_SA_NOT_OFFERED: Remote peer % 15i responded with attribute [chars] not offered or changed. - %CRYPTO-6-IKMP_SA_NOT_AUTH: Cannot accept Quick Mode exchange from %15i if SA is not authenticated! - IPSec(validate_transform_proposal): proxy identities not supported; ISAKMP: IPSec policy invalidated proposal - IPSEC(validate_proposal): invalid local address IP_address |  |

Answers

| 8. | -  B. QM_IDLE indicates that the management connection has completed successfully. -  A indicates that the tunnel has been initialized but nothing has been negotiated. C indicates that the IPSec peers have negotiated the ISAKMP/IKE Phase 1 policies. D indicates that the management SA has been authenticated. |
| --- | --- |
| 9. | -  A. The "%CRYPTO-6-IKMP_SA_NOT_OFFERED: Remote peer % 15i responded with attribute [chars] not offered or changed." message in the output of the debug crypto isakmp command indicates a failed policy negotiation. -  B indicates Phase 1 authentication failure. C indicates a mismatched crypto ACL in Phase 2. D indicates an invalid remote peer IP address in Phase 1. |

#### Classic Method Site-to-Site Example

| 10. | What crypto map command is used to specify the crypto ACL to use? - set transform-set - match address - set address - set crypto-acl |  |
| --- | --- | --- |

Answers

| 10. | -  B. The match address crypto map command is used to specify the crypto ACL to use. -  A defines the transform set to use. C and D are invalid commands. |
| --- | --- |